Project: Springfield Municipal Services Infrastructure

Document: Network Architecture & Device Inventory Plan

Date: January 23, 2026

Team 6: Cameron, Fiston, Rita, Cayla

1. Device Naming Convention & Inventory

All network hardware strictly adheres to the standard format: SITE-ROLE-NUM.

- Site Identifier: SPF (Springfield)

- Role Identifiers: FW (Firewall), CSW (Core Switch), ASW (Access Switch), SRV (Server)

Hardware Inventory:

- SPF-FW-01: Cisco ASA Firewall | Role: Perimeter Security, NAT, & VPN Termination.

- SPF-CSW-01: Layer 3 Core Switch | Role: Inter-VLAN Routing & DHCP Services.

- SPF-ASW-01: Layer 2 Access Switch | Role: Public Safety Access (VLAN 10).

- SPF-ASW-02: Layer 2 Access Switch | Role: Municipal Admin Access (VLAN 20).

- SPF-ASW-03: Layer 2 Access Switch | Role: Public WiFi Access (VLAN 50).

2. Master IP Addressing & VLAN Plan

Addressing relies on Variable Length Subnet Masking (VLSM) to optimize address space and enforce strict zone boundaries.

| VLAN ID | VLAN Name | Security Zone | Subnet (CIDR) | Gateway | DHCP Scope |
| --- | --- | --- | --- | --- | --- |
| 10 | Public Safety | CJIS (High) | 10.60.10.0/24 | 10.60.10.1 | 10.60.10.50 – .254 |
| 20 | Muni Admin | Internal (Med) | 10.60.20.0/24 | 10.60.20.1 | 10.60.20.50 – .254 |
| 30 | Management | Restricted | 10.60.30.0/26 | 10.60.30.1 | Static Only |
| 40 | Citizen DMZ | DMZ (Low) | 10.60.40.0/26 | 10.60.40.1 | Static Only |
| 50 | Public WiFi | Untrusted | 10.60.50.0/23 | 10.60.50.1 | 10.60.50.11 – 51.254 |

3. Topology Verification Evidence

(Documentation Note: Capture a screenshot of the Packet Tracer environment. Ensure all devices are labeled according to the SPF-XXX-01 naming convention outlined above. Save the artifact to the /screens repository.)