Project: Springfield Municipal Services Infrastructure

Document: Layer 2 Port Assignment & Verification Log

Date: February 2, 2026

Team 6: Cameron, Fiston, Rita, Cayla

1. Port Assignment & Security Table

A. Core Switch (SPF-CSW-01) - Distribution & Inter-VLAN Routing

| Interface | Type | VLANs Allowed | Connected Device | Security / Notes |
| --- | --- | --- | --- | --- |
| Gi1/0/1 | Trunk | 10, 20, 30, 40, 50 | SPF-FW-01 (Firewall) | Uplink to Gateway. Native VLAN 99. |
| Gi1/0/2 | Trunk | 10, 30 | SPF-ASW-01 (Public Safety) | Pruned: Only Safety + Mgmt allowed. |
| Gi1/0/3 | Trunk | 20, 30 | SPF-ASW-02 (City Admin) | Pruned: Only Admin + Mgmt allowed. |
| Gi1/0/4 | Trunk | 50, 30 | SPF-ASW-03 (Public WiFi) | Pruned: Only Guest + Mgmt allowed. |
| Gi1/0/5 | Access | 40 | DMZ-Server-Switch | Isolated: DMZ traffic only. |

Export to Sheets

B. Public Safety Access Switch (SPF-ASW-01) - High Security

| Interface | Type | VLAN ID | Connected Device | Port Security Configuration |
| --- | --- | --- | --- | --- |
| Gi0/1 | Trunk | 10, 30 | SPF-CSW-01 (Uplink) | Trusted Uplink (No Port Security). |
| Fa0/1–Fa0/12 | Access | 10 | Police Workstations | Max-Mac: 2, Violation: Shutdown, Sticky |
| Fa0/13–Fa0/24 | Access | 99 | Unused / Parking | Administratively Down (shutdown) |

Export to Sheets

2. Layer 2 Implementation Scripts

Saved in repository /scripts.

File: SPF-CSW-01_VLAN_Script.txt

Plaintext

! Springfield Core Switch VLAN Configuration

vlan 10

name PUBLIC-SAFETY

vlan 20

name MUNI-ADMIN

vlan 30

name MANAGEMENT

vlan 40

name DMZ-SERVICES

vlan 50

name PUBLIC-WIFI

vlan 99

name PARKING-NATIVE

exit

interface range gi1/0/2-4

description Downlinks-to-Access

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

no shutdown

exit

! Trunk Pruning

interface gi1/0/2

switchport trunk allowed vlan 10,30

interface gi1/0/3

switchport trunk allowed vlan 20,30

interface gi1/0/4

switchport trunk allowed vlan 50,30

exit

File: SPF-ASW-01_Security_Script.txt

Plaintext

! Springfield Public Safety Switch Security Configuration

interface range fa0/1-12

description POLICE-WORKSTATION

switchport mode access

switchport access vlan 10

spanning-tree portfast

switchport port-security

switchport port-security maximum 2

switchport port-security violation shutdown

switchport port-security mac-address sticky

no shutdown

exit

! Disable Unused Ports

interface range fa0/13-24

description UNUSED

switchport access vlan 99

shutdown

exit

3. Verification Evidence Log

(Documentation Note: The following CLI outputs confirm successful Layer 2 deployment. Artifacts are generated in collaboration with the Network Lead.)

- VLAN Verification (Task 11): * Command: show vlan brief on SPF-CSW-01.

  - Success Criteria: Verifies VLANs 10, 20, 30, 40, 50 exist and are named correctly.

  - Artifact: [Insert CLI Output Here]

- Trunk Verification (Task 12):

  - Command: show interfaces trunk on SPF-CSW-01.

  - Success Criteria: Verifies trunk allowances match the assignment table.

  - Artifact: [Insert CLI Output Here]

- Port Security Verification (Task 13):

  - Command: show port-security interface fa0/1 on SPF-ASW-01.

  - Success Criteria: Confirms Port Security is Enabled and Violation Mode is Shutdown.

  - Artifact: [Insert CLI Output Here]