Project: Springfield Municipal Services Infrastructure

Document: Routing Validation, Performance, & Resilience Report

Date: February 23, 2026

Team 6: Cameron, Fiston, Rita, Cayla

1. Department Access Validation

| Source | Destination | Protocol | Expected Result | Actual Result |
| --- | --- | --- | --- | --- |
| Admin PC (VLAN 20) | Safety PC (VLAN 10) | ICMP | Success | SUCCESS |
| Safety PC (VLAN 10) | DMZ Web (VLAN 40) | HTTPS | Success | SUCCESS |
| Public WiFi (VLAN 50) | Safety PC (VLAN 10) | ICMP | Fail | FAIL (Isolated) |
| Safety PC (VLAN 10) | 911 Dispatch (VLAN 10) | ICMP | Success | SUCCESS |

Export to Sheets

2. Performance Baseline

- Test: Traceroute from Public Safety PC (10.60.10.10) to DMZ Server (10.60.40.10).

- Path: PC 10.60.10.1 (Core Gateway) 10.60.40.10 (DMZ).

- Baseline Latency: < 1ms average latency across 50 ICMP packets.

3. Network Rebuild / Failover Testing

- Scenario: The primary uplink from SPF-ASW-01 to SPF-CSW-01 fails.

- Execution: shutdown executed on SPF-CSW-01 interface Gi1/0/2.

- Result: Spanning Tree Protocol successfully recalculated the path. The backup link converged in < 30 seconds, and traffic resumed normally.

- Rebuild Readiness: Base configurations have been backed up to the /configs repository. Total disaster recovery bare-metal restore time is estimated at < 2 hours utilizing the automated deployment scripts.