Project: Springfield Municipal Services Infrastructure

Document: ASA Perimeter Security & Network Address Translation (NAT)

Date: March 16, 2026

Team 6: Cameron, Fiston, Rita, Cayla

1. Firewall Security Zones & Posture

The perimeter Cisco ASA (SPF-FW-01) is configured with four distinct security zones. By default, the ASA explicitly denies traffic initiating from lower security levels to higher security levels unless permitted by an ACL.

| Interface | Zone Name | Security Level | Connected Subnet | Trust Posture |
| --- | --- | --- | --- | --- |
| Gig1/1 | OUTSIDE | 0 | ISP / Internet | Untrusted. All unsolicited inbound traffic denied. |
| Gig1/2 | DMZ | 50 | 10.60.40.0/26 | Low Trust. Hosts public-facing Citizen Portal. |
| Gig1/3 | INSIDE | 80 | 10.60.20.0/24 | Medium Trust. Municipal Admin operations. |
| Gig1/4 | CJIS | 100 | 10.60.10.0/24 | High Trust. Strictly protected per FBI mandate. |

Export to Sheets

2. CJIS Zone Boundary Protection

By assigning the CJIS zone the maximum security level of 100, the firewall inherently drops all unauthorized connection attempts from the INSIDE, DMZ, and OUTSIDE networks. This architecture satisfies FBI CJIS Policy Area 5.5 by enforcing strict logical boundary protection around the Public Safety environment.

3. Secured Management & NAT

- Management Access: Administrative access to SPF-FW-01 is strictly controlled via SSHv2. Telnet is disabled. Access is exclusively restricted to the dedicated Management Subnet (10.60.30.0/26).

- Network Address Translation (NAT): To provide secure internet access, Dynamic Port Address Translation (PAT) maps internal network traffic to the firewall's public-facing IP.

  - Rule: nat (INSIDE,OUTSIDE) dynamic interface. This ensures internal users can browse the web while external threats cannot map internal municipal IP addresses.