Project: Springfield Municipal Services Infrastructure

Document: Audit Compliance Matrix & Exceptions Register

Date: March 30, 2026

Team 6: Cameron, Fiston, Rita, Cayla

1. Compliance Control Matrix

| Compliance Standard | Requirement / Scope | Technical Mechanism | Test / Validation Method | Evidence Artifact Link |
| --- | --- | --- | --- | --- |
| CJIS 5.5 (Access Control) | Strict boundary protection for Public Safety VLAN (Task 51). | ASA Firewall Zone (Security Level 100) + ACL-05 to ACL-08. | Packet-tracer simulation denying inbound traffic. | FW1_packet-tracer_CJIS_Deny.txt. |
| CJIS 5.4 (Auditing) | Log all access attempts to CJIS data. | AD Group Policy: Audit Logon Events (Success/Failure). | gpresult /r and Windows Event Viewer logs. | DC01_Audit_Policy_Log.png. |
| CJIS 5.6 (Authentication) | Complex passwords for law enforcement. | AD Password Policy (14 chars, 90-day reset) applied to Public Safety OU. | GPO Settings Export. | GPO_CJIS_Password_Policy.html. |
| Public Records / FOIA | Maintain available public records access (Task 52). | DMZ Web Server with Read-Only backend SQL access. | HTTP test from Outside to DMZ. | FW1_packet-tracer_DMZ_Allow.txt. |
| ADA / Sec 508 | Citizen Portal accessibility (Task 53). | Web portal structured for screen readers (Simulated requirement). | Architecture documentation note. | See Exceptions Register. |

Export to Sheets

2. Compliance Exceptions Register

| Control | Exception Description | Reason | Compensating Control | Risk Owner |
| --- | --- | --- | --- | --- |
| ADA/508 | Cannot fully validate WCAG screen-reader compliance via Packet Tracer. | PT limitation; lacks application-layer rendering. | DMZ server placed on isolated VLAN to ensure network stability while awaiting app-level code review. | Project Manager |
| CJIS 5.13 (Mobile) | MDM (Mobile Device Management) for police cruisers not implemented. | Out of scope for Week 11 lab environment. | MAC address sticky port-security limits unauthorized devices on physical ports. | Security Lead |

Export to Sheets

3. Evidence Artifact Index

- /configs/2026-03-29_FW1_running-config.txt Proves ACLs and NAT configuration (Tasks 46-48).

- /screenshots/2026-03-29_DC01_Audit_Policy_Log.png Proves CJIS 5.4 Auditing (Task 51).

- /reports/GPO_CJIS_Password_Policy.html Proves CJIS 5.6 Authentication.

- /logs/2026-03-29_FW1_packet-tracer_CJIS_Deny.txt Proves CJIS boundary isolation.

- /logs/2026-03-29_FW1_show_nat.txt Proves DMZ routing for Public Records.