Project: Springfield Municipal Services Infrastructure

Document: Layer 2 Verification & Troubleshooting Standard Operating Procedure (SOP)

Date: February 9, 2026

Team 6: Cameron, Fiston, Rita, Cayla

1. Spanning Tree Protocol (STP) Topology & Strategy

To prevent Layer 2 bridging loops and ensure highly available, optimal traffic flow, the network utilizes a deterministic Spanning Tree Protocol architecture. The Core Switch (SPF-CSW-01) has been manually designated as the Root Bridge for all operational VLANs.

STP Status Matrix:

| Device Name | Hardware Role | STP Priority | Root Bridge? | Operational Status |
| --- | --- | --- | --- | --- |
| SPF-CSW-01 | Core/Distribution | 24576 | YES | Forwarding on all connected trunks. |
| SPF-ASW-01 | Access (Safety) | 32768 (Default) | NO | Root Port: Gi0/1 (Forwarding). |
| SPF-ASW-02 | Access (Admin) | 32768 (Default) | NO | Root Port: Gi0/1 (Forwarding). |
| SPF-ASW-03 | Access (WiFi) | 32768 (Default) | NO | Root Port: Gi0/1 (Forwarding). |

Export to Sheets

CJIS Compliance Note: To satisfy security and environmental stability requirements, unused ports on all access switches are configured with spanning-tree bpduguard enable. This security control prevents rogue or unauthorized switches from being introduced into the environment and attempting to hijack the STP Root role.

2. Tier 1 Troubleshooting Guide (Layer 2)

This living document outlines standard remediation procedures for common Layer 2 connectivity failures.

Issue 1: Workstation cannot communicate with devices in the SAME VLAN.

- Potential Cause: The access port is assigned to the incorrect VLAN or is administratively down.

- Validation Step: Execute show vlan brief on the designated access switch. Verify physical link status indicators.

- Resolution: Enter interface configuration mode and explicitly apply switchport access vlan [ID]. Verify the port state using no shutdown.

Issue 2: Traffic fails to pass between the Access Switch and Core Switch.

- Potential Cause: Trunking mismatch (e.g., Native VLAN mismatch across the link or allowed VLANs pruned incorrectly on one side).

- Validation Step: Execute show interfaces trunk on both terminating ends of the uplink. Check for STP blocking states.

- Resolution: Ensure switchport trunk native vlan 99 is configured identically on both interfaces. Verify the required VLAN ID is present in the allowed list.