Project: Springfield Municipal Services Infrastructure

Document: Inter-VLAN Routing & Quality of Service (QoS) Architecture

Date: February 16, 2026

Team 6: Cameron, Fiston, Rita, Cayla

1. Layer 3 Switch Virtual Interface (SVI) Configurations

The following SVIs have been configured on the Core Switch (SPF-CSW-01) to act as default gateways for inter-department routing.

- VLAN 10 (Public Safety): 10.60.10.1 /24

- VLAN 20 (Muni Admin): 10.60.20.1 /24

- VLAN 30 (Management): 10.60.30.1 /26

- VLAN 40 (DMZ): 10.60.40.1 /26

- VLAN 50 (Public WiFi): 10.60.50.1 /23

2. Default Routing (State/Federal Connection)

A static default route has been configured to forward all unknown external traffic to the Edge Firewall (SPF-FW-01).

- ip route 0.0.0.0 0.0.0.0 10.60.30.2 (Forwards to Firewall Inside IP)

3. Emergency Services Priority (QoS)

To ensure 911 Dispatch and Public Safety communications are not interrupted by network congestion, strict Quality of Service (QoS) boundaries have been established. Traffic originating from the Public Safety VLAN (VLAN 10) is marked with a DSCP value of Expedited Forwarding (EF). The Core Router places EF-marked traffic into a strict priority queue, guaranteeing bandwidth during high-load municipal events.

4. Connectivity Matrix (Week 5 Status)

(Note: Full ACL restrictions will be applied in Week 10. This matrix proves baseline Layer 3 functionality.)

| Source | Destination | Ping Result | Protocol Used |
| --- | --- | --- | --- |
| Admin PC (VLAN 20) | Default Gateway (10.60.20.1) | SUCCESS | ICMP |
| Admin PC (VLAN 20) | Safety PC (VLAN 10) | SUCCESS | Inter-VLAN Routing |
| Safety PC (VLAN 10) | Default Gateway (10.60.10.1) | SUCCESS | ICMP |
| Public WiFi (VLAN 50) | Safety PC (VLAN 10) | BLOCKED | L3 Isolation |